Yeast Fermentation Activity

The fermentation of glucose can be described by the following equation:

C6H12O6  2 CH3CH2OH + 2 CO2 + energy

glucose	ethanol	carbon dioxide

OBJECTIVES

In this experiment, you will use a Gas Pressure Sensor to measure the pressure change caused by CO2 production during anaerobic fermentation.

MATERIALS

| TI-84 Plus graphing calculator | test tube |
| --- | --- |
| EasyData application Data-collection interface (EasyLink) Pressure Sensor | sucrose sugar solution yeast suspension vegetable oil |
| utility clamp rubber-stopper assembly syringe or graduated cylinder | Pipette or eyedropper |

PROCEDURE

1.	Connect the plastic tubing to the valve on the Pressure Sensor.

- 2.	Turn on the calculator. Connect the Pressure Sensor, data-collection interface, and calculator.  (With EasyLink the calculator will automatically launch EasyData  and detect the sensor.)

3.	Set up EasyData for data collection.

- Start the EasyData application if it is not already running.

- Select  from the Main screen, and then select New to reset the application.

4.	Obtain a plastic test tube.

5.	Place 10 mL of the sucrose sugar solution into the test tube

6.	Obtain the yeast suspension. Gently swirl the yeast suspension to mix the yeast that settles to the bottom. Using a syringe or graduated cylinder, transfer 5 mL of yeast suspension into the test tube. Gently mix the yeast into the sugar solution.

7.	In the test tube, place enough vegetable oil to completely cover the surface of the yeast/glucose mixture.  Be careful to not get oil on the inside wall of the test tube.

8.	Insert the rubber-stopper into the test tube. Note: Firmly twist the stopper for an airtight fit. Place the test tube in the plastic glass so you do not need to hold it.

Figure 2

9.	Connect the free-end of the plastic tubing to the connector in the rubber stopper.  See diagram to the right.

10.	 Select  to begin data collection.

11.		Data collection will end after 15 minutes. Monitor the pressure readings displayed on the calculator screen. If the pressure exceeds 130 kilopascals, the pressure inside the tube will be too great and the rubber stopper is likely to pop off. Disconnect the plastic tubing from the Pressure Sensor if the pressure exceeds 130 kilopascals.

12.	 When data collection has finished, an auto-scaled graph of pressure vs. time will be displayed on the calculator screen. As you move the cursor right or left, the time (X) and pressure (Y) values of each data point are displayed above the graph.

13.	Disconnect the plastic tubing connector from the rubber stopper. Remove the rubber stopper from the test tube and discard the contents in a waste container.

14.	Determine the rate of fermentation for the curve of pressure vs. time. The rate of fermentation can be measured by examining the slope of the pressure vs. time curve:

| - Examine the graph and determine the most linear region. - Move the flashing cursor to the first point of the region. Enter the initial pressure, Pi, and initial time, ti, in Table 1. - Move the flashing cursor to the last point of the region. Enter the final pressure, Pf, and final time, tf, in Table 1. |  |
| --- | --- |

16.	Select  to return to the Main screen.

17.	Select , then  to exit EasyData

PROCESSING THE DATA

Calculate the slope of the graph for both of the experimental run. The slope of each graph is equal to the rate of fermentation for that experiment. Record the calculated slope

QUESTIONS

1.	Why do you need to incubate the yeast before you start monitoring air pressure?

2.	Hypothesize how other sugars as glucose, fructose and maltose would affect the results. .

3.	How would temperature affect the rate of respiration?  Is there a limit to how warm

the temperature can be for the yeast to carry on fermentation?  Why?

Setup Notes:

Sugar packets from restaurants have 1 teaspoon or about 3.5 g of sucrose

A 10% sucrose solution works well.

Quick rise yeast works well – 1pkg per 200 mL for the yeast suspension.